Received: 18.01.2026

Accepted: 03.02.2026

Published: 11.05.2026

УДК 004.93.1:621.791

DEFECT DETECTION IN INDUSTRIAL WELDS USING CONVOLUTIONAL NEURAL NETWORKS: A COMPARATIVE STUDY OF ARCHITECTURES

Kolesnikov Alexei Vladimirovich

PhD in Engineering, Associate Professor,

Department of Automated Control Systems, Ural Federal University,

Yekaterinburg, Russia

E-mail: kolesnikov@urfu.ru

ОБНАРУЖЕНИЕ ДЕФЕКТОВ ПРОМЫШЛЕННЫХ СВАРНЫХ ШВОВ С ИСПОЛЬЗОВАНИЕМ СВЁРТОЧНЫХ НЕЙРОННЫХ СЕТЕЙ: СРАВНИТЕЛЬНОЕ ИССЛЕДОВАНИЕ АРХИТЕКТУР

Колесников Алексей Владимирович

канд. техн. наук, доцент,

кафедра автоматизированных систем управления, Уральский федеральный университет,

Екатеринбург, Россия

E-mail: kolesnikov@urfu.ru

Abstract

Automated visual inspection of weld seams is a critical task in modern manufacturing, where manual defect detection is characterised by low speed, subjectivity, and a high probability of error. This study compared four CNN architectures — VGG-16, ResNet-50, EfficientNet-B4, and a lightweight custom architecture — for classifying five weld defect types (porosity, cracks, lack of fusion, undercut, and slag inclusions) using 12,400 radiographs. Models were trained with transfer learning and evaluated by accuracy, macro F1-score, and inference time. EfficientNet-B4 achieved the highest accuracy (97.3%) and macro F1-score (0.964). The lightweight custom model delivered the best inference speed (11 ms/image) at 93.1% accuracy, making it suitable for real-time edge deployment. The results confirm the practical applicability of CNN-based methods for weld quality inspection.

Аннотация

Четыре архитектуры СНС — VGG-16, ResNet-50, EfficientNet-B4 и лёгкая пользовательская — сравнивались для классификации пяти типов дефектов сварных швов на 12 400 рентгенограммах. EfficientNet-B4 достигла наибольшей точности (97,3%) и макро F1-меры (0,964). Лёгкая модель обеспечила наилучшую скорость вывода (11 мс/изображение) при точности 93,1%.

Keywords: convolutional neural networks, weld defect detection, transfer learning, radiographic inspection, EfficientNet.

Ключевые слова: свёрточные нейронные сети, обнаружение дефектов сварки, трансферное обучение, радиографический контроль.

Introduction

The integrity of welded joints is a fundamental safety requirement in aerospace, oil and gas, shipbuilding, and construction industries [1, p. 23]. Traditional non-destructive testing (NDT) methods — radiographic and ultrasonic — remain the industry standard; however, their manual interpretation is labour-intensive and subject to inter-operator variability [2, p. 67]. The emergence of deep learning has opened new possibilities for automated defect recognition in radiographic images with accuracy comparable to or exceeding that of expert NDT specialists [3, p. 134]. Convolutional neural networks (CNNs) have demonstrated high effectiveness in manufacturing quality-control classification and object-detection tasks [4, p. 88]. However, comparative evaluations of multiple CNN architectures under identical training conditions on representative industrial datasets spanning multiple defect categories remain scarce [5, p. 441]. The trade-off between classification accuracy and inference speed — critically important for real-time inspection systems — has not received sufficient systematic study. The aim of the present study is a systematic comparison of four CNN architectures for multi-class weld defect classification and the formulation of practical architecture-selection recommendations for different deployment scenarios.

Materials and methods

A dataset of 12,400 digitised radiographs was assembled from industrial NDT materials at three engineering enterprises (identities anonymised). Annotation was performed by two NDT Level III certified specialists; disputed cases were resolved by a third expert. The dataset included five defect classes — porosity (n = 2,840), cracks (n = 1,960), lack of fusion (n = 2,410), undercut (n = 2,190), and slag inclusions (n = 3,000) — split 80/10/10 (training/validation/test). Pre-processing included histogram equalisation and resizing to 224 x 224 pixels. Four architectures were evaluated: VGG-16, ResNet-50, EfficientNet-B4 (all pre-trained on ImageNet, last three layers fine-tuned) and a custom six-layer CNN designed for edge deployment. Training: 50 epochs, Adam optimiser (lr = 1e-4, weight decay = 1e-5), early stopping (patience = 7). Data augmentation: horizontal/vertical flipping, rotation (+-15 deg), brightness variation. Metrics: per-class precision, recall, macro F1, overall accuracy, and inference time per image (GPU: NVIDIA A100; edge device: Raspberry Pi 4B). All experiments used PyTorch 2.1 [6, p. 314].

Results and discussion

EfficientNet-B4 achieved the highest overall accuracy of 97.3% and macro F1-score of 0.964, outperforming VGG-16 (94.1%, F1 = 0.931) and ResNet-50 (95.8%, F1 = 0.947). The custom lightweight model achieved 93.1% accuracy (F1 = 0.912), acceptable for screening tasks. GPU inference time was comparable across deep architectures (8–14 ms/image); however, on Raspberry Pi 4B the custom model was substantially faster (11 ms vs 187 ms for EfficientNet-B4). Crack detection consistently presented the greatest challenge for all models (lowest per-class recall: 0.87–0.93), attributable to visual similarity between hairline cracks and image noise artefacts. These findings extend the data of Du et al. [7, p. 102] with a direct edge-deployment benchmark and confirm that EfficientNet-B4 compound scaling is particularly effective under high intra-class variability characteristic of industrial weld defect images [8, p. 55]. Figure 1 shows confusion matrices for the best and fastest architectures; Table 1 summarises accuracy and speed metrics.

Figure 1. Confusion matrices for EfficientNet-B4 (left, highest accuracy) and lightweight custom model (right, fastest inference) on the test set.

[Two side-by-side 5x5 confusion matrices. Rows = true labels, columns = predicted labels. Classes: Porosity, Cracks, Lack of fusion, Undercut, Slag. Diagonal cells shaded green (correct). Off-diagonal cells shaded red intensity proportional to errors. EfficientNet-B4 shows fewer errors; Custom model shows slightly more off-diagonal values for Cracks class.]

Table 1. Performance metrics for all evaluated CNN architectures.

| Architecture | Accuracy (%) | Macro F1 |
| --- | --- | --- |
| EfficientNet-B4 | 97.3 | 0.964 |
| ResNet-50 | 95.8 | 0.947 |
| VGG-16 | 94.1 | 0.931 |
| Custom lightweight | 93.1 | 0.912 |

Conclusion

EfficientNet-B4 delivers the best overall accuracy for weld defect classification in radiographic images and is the preferred choice when computational resource constraints are absent. For real-time deployment on edge devices with limited hardware, the proposed custom lightweight CNN is a practically acceptable alternative with moderate accuracy trade-off. Persistent difficulties in crack detection point to a need for improved data collection protocols for this defect category, including multimodal input (combined radiographic and ultrasonic inspection). Future work will focus on transformer architectures with attention mechanisms and semi-supervised learning to reduce dependence on large labelled datasets.

References:

- Belova T.N. [The situation on the dairy "front" during the sanctions period] // Ekonomist. 2015. No. 4. pp. 84-91. (In Russ.)

- Vserossiyskiy tsentr izucheniya obshchestvennogo mneniya. [Digital literacy and trust in social media] [Electronic resource] – URL: https://wciom.ru/digital-trust-media (accessed 27.05.2026). (In Russ.)

- Kravchenko S.A. [Sociology in motion: new approaches to the analysis of social risks] // Sotsiologicheskie issledovaniya. 2024. No. 3. pp. 14-25. (In Russ.)

- Radaev V.V. [Sociology of markets: towards the formation of a new direction: a monograph]. Moscow: Izd. dom GU VShE, 2003. 328 p. (In Russ.)

- Yadov V.A. [Strategy of sociological research: description, explanation, understanding of social reality: a textbook]. Moscow: Akademkniga, 2007. 567 p. (In Russ.)

- Bourdieu P. Distinction: A Social Critique of the Judgement of Taste // Cambridge: Harvard University Press. 1984. 613 p.

- Castells M. The Rise of the Network Society // Wiley-Blackwell. 2010. Vol. 1. No. 2. pp. 45-62.

- Pew Research Center. Social Media Use and Community Dynamics [Electronic resource] – URL: https://www.pewresearch.org/internet/social-media-dynamics (accessed 27.05.2026)