Received: 18.01.2026

Accepted: 03.02.2026

Published: 11.05.2026

Educational example for Universum: Technical Sciences. Author details, dates, funding, and research results are illustrative.

UDC 004.93.1:621.791

Defect detection in industrial welds using convolutional neural networks: A comparative study of architectures

Kolesnikov Alexei Vladimirovich

PhD in Engineering, Associate Professor,

Department of Automated Control Systems, Ural Federal University,

Yekaterinburg, Russia

E-mail: kolesnikov@urfu.ru

ORCID: 0000-0001-2345-6789

RSCI SPIN code: 1234-5678

Обнаружение дефектов промышленных сварных швов с использованием свёрточных нейронных сетей: сравнительное исследование архитектур

Колесников Алексей Владимирович

канд. техн. наук, доцент,

кафедра автоматизированных систем управления, Уральский федеральный университет,

Екатеринбург, Россия

E-mail: kolesnikov@urfu.ru

ORCID: 0000-0001-2345-6789

SPIN-код: 1234-5678

Abstract

Automated visual inspection of weld seams is a critical task in modern manufacturing, where manual defect detection is characterized by low speed, subjectivity, and a high probability of error. Convolutional Neural Networks (CNNs) demonstrate significant potential for automating this process. In this study, four CNN architectures—VGG-16, ResNet-50, EfficientNet-B4, and a lightweight custom architecture—were compared for the classification of five types of weld defects (porosity, cracks, lack of fusion, undercut, and slag inclusions) using a dataset of 12,400 radiographs. The models were trained using transfer learning and evaluated based on accuracy, F1-score, and inference time. EfficientNet-B4 achieved the highest accuracy (97.3%) and macro F1-score (0.964). The lightweight custom model delivered the best inference speed (11 ms per image) with an accuracy of 93.1%, making it suitable for real-time systems. These results confirm the practical applicability of CNNs for weld quality inspection. The comparison considers recognition quality together with processing speed, because architecture choice depends on available computing resources and inspection requirements. The results distinguish the priority of maximum accuracy from the need to operate on an edge device. Cracks remain the most difficult category. The practical contribution is an evidence-based choice of architecture for a specific deployment scenario. Generalisation to new industrial sites requires further evaluation using independent data.

Аннотация

Автоматизированный визуальный контроль качества сварных швов является критически важной задачей в современном производстве, где ручное обнаружение дефектов отличается низкой скоростью, субъективностью и высокой вероятностью ошибок. Свёрточные нейронные сети (СНС) демонстрируют высокий потенциал для автоматизации данного процесса. В настоящем исследовании сравнивались четыре архитектуры СНС — VGG-16, ResNet-50, EfficientNet-B4 и лёгкая пользовательская архитектура — для классификации пяти типов дефектов сварных швов (пористость, трещины, непровар, подрез и шлаковые включения) на наборе из 12 400 рентгенограмм. Модели обучались с применением трансферного обучения и оценивались по точности, F1-мере и времени вывода. EfficientNet-B4 достигла наибольшей точности (97,3%) и макро F1-меры (0,964). Пользовательская лёгкая модель обеспечила наилучшую скорость вывода (11 мс/изображение) при точности 93,1%, что делает её пригодной для систем реального времени. Результаты подтверждают практическую применимость СНС для контроля качества сварки. Сопоставление выполнялось по совокупности показателей качества распознавания и скорости обработки, поскольку выбор архитектуры зависит от вычислительных ресурсов и требований производственного контроля. Результаты показывают различие между приоритетом максимальной точности и необходимостью работы на периферийном устройстве. Наиболее сложной категорией остаются трещины. Практическое значение исследования состоит в обосновании выбора модели для конкретного сценария развёртывания. Переносимость полученных оценок на новые предприятия требует дополнительной проверки на независимых данных.

Keywords: convolutional neural networks, weld defects, transfer learning, radiographic inspection, EfficientNet, edge computing.

Ключевые слова: свёрточные нейронные сети, дефекты сварных швов, трансферное обучение, радиографический контроль, EfficientNet, периферийные вычисления.

Funding: The study was supported by the Russian Science Foundation (RSF), grant No. 24-19-00347 “Intelligent Systems for Automated Quality Control of Welded Joints.”

Финансирование: Исследование выполнено при финансовой поддержке Российского научного фонда (РНФ), грант № 24-19-00347 «Интеллектуальные системы автоматизированного контроля качества сварных соединений».

Conflict of interest: The author declares no conflict of interest (illustrative wording).

Конфликт интересов: автор заявляет об отсутствии конфликта интересов (учебный пример формулировки).

Acknowledgements: ChatGPT (OpenAI) was used to translate, edit, and harmonise the formatting of this educational example. Numerical data were retained from the original example. The tool was not used to produce research findings.

Благодарности: при обновлении данного учебного примера использован ChatGPT (OpenAI) для перевода, языковой редакции и согласования оформления; числовые данные сохранены из исходного примера. Инструмент не использовался для получения исследовательских результатов.

Introduction

Deep learning and transfer learning provide a methodological basis for image classification [1, 3]. Architecture comparisons depend on the input datasets, preprocessing, and training procedures [4, 5, 7, 8]. This example compares VGG, ResNet, and EfficientNet families [2, 9, 10], while transformer approaches provide a direction for future work [6]. The study compares the quality and speed of weld defect classification using four architectures under consistent conditions on an illustrative dataset of 12,400 radiographs.

Materials and methods

A dataset of 12,400 digitised radiographs was assembled from industrial NDT materials at three engineering enterprises (identities anonymised). Annotation was performed by two NDT Level III certified specialists; disputed cases were resolved by a third expert. The dataset included five defect classes — porosity (n = 2,840), cracks (n = 1,960), lack of fusion (n = 2,410), undercut (n = 2,190), and slag inclusions (n = 3,000) — split 80/10/10 (training/validation/test). Pre-processing included histogram equalisation and resizing to 224 x 224 pixels. Four architectures were evaluated: VGG-16, ResNet-50, EfficientNet-B4 (all pre-trained on ImageNet, last three layers fine-tuned) and a custom six-layer CNN designed for edge deployment. Training: 50 epochs, Adam optimiser (lr = 1e-4, weight decay = 1e-5), early stopping (patience = 7). Data augmentation: horizontal/vertical flipping, rotation (+-15 deg), brightness variation. Metrics: per-class precision, recall, macro F1, overall accuracy, and inference time per image (GPU: NVIDIA A100; edge device: Raspberry Pi 4B). All experiments used PyTorch 2.1.

Results and discussion

EfficientNet-B4 achieved the highest overall accuracy of 97.3% and macro F1-score of 0.964, outperforming VGG-16 (94.1%, F1 = 0.931) and ResNet-50 (95.8%, F1 = 0.947). The custom lightweight model achieved 93.1% accuracy (F1 = 0.912), acceptable for screening tasks. GPU inference time was comparable across deep architectures (8–14 ms/image); however, on Raspberry Pi 4B the custom model was substantially faster (11 ms vs 187 ms for EfficientNet-B4). Crack detection consistently presented the greatest challenge for all models (lowest per-class recall: 0.87–0.93), attributable to visual similarity between hairline cracks and image noise artefacts. Figure 1 presents the main comparative values. Table 1 summarises classification metrics.

Figure 1. Defect classification accuracy for four architectures

Table 1. Classification metrics for the compared architectures

| Architecture | Accuracy (%) | Macro F1 |
| --- | --- | --- |
| EfficientNet-B4 | 97.3 | 0.964 |
| ResNet-50 | 95.8 | 0.947 |
| VGG-16 | 94.1 | 0.931 |
| Custom lightweight | 93.1 | 0.912 |

Note: data from the educational example.

Conclusion

EfficientNet-B4 delivers the best overall accuracy for weld defect classification in radiographic images and is the preferred choice when computational resource constraints are absent. For real-time deployment on edge devices with limited hardware, the proposed custom lightweight CNN is a practically acceptable alternative with moderate accuracy trade-off. Persistent difficulties in crack detection point to a need for improved data collection protocols for this defect category, including multimodal input (combined radiographic and ultrasonic inspection). Future work will focus on transformer architectures with attention mechanisms and semi-supervised learning to reduce dependence on large labelled datasets.

References:

1. LeCun Y., Bengio Y., Hinton G. Deep learning // Nature. – 2015. – Vol. 521. – No. 7553. – pp. 436–444. – DOI: 10.1038/nature14539.

2. He K., Zhang X., Ren S., Sun J. Deep Residual Learning for Image Recognition // 2016 IEEE Conference on Computer Vision and Pattern Recognition (CVPR). – 2016. – pp. 770–778. – DOI: 10.1109/cvpr.2016.90.

3. Pan S.J., Yang Q. A Survey on Transfer Learning // IEEE Transactions on Knowledge and Data Engineering. – 2010. – Vol. 22. – No. 10. – pp. 1345–1359. – DOI: 10.1109/tkde.2009.191.

4. Deng J., Dong W., Socher R., Li L.J. et al. ImageNet: A large-scale hierarchical image database // 2009 IEEE Conference on Computer Vision and Pattern Recognition. – 2009. – pp. 248–255. – DOI: 10.1109/cvpr.2009.5206848.

5. Mery D., Riffo V., Zscherpel U., Mondragón G. et al. GDXray: The Database of X-ray Images for Nondestructive Testing // Journal of Nondestructive Evaluation. – 2015. – Vol. 34. – No. 4. – Article 42. – DOI: 10.1007/s10921-015-0315-7.

6. Dosovitskiy A., Beyer L., Kolesnikov A. An Image is Worth 16x16 Words: Transformers for Image Recognition at Scale // International Conference on Learning Representations. – 2021. – URL: https://arxiv.org/abs/2010.11929 (accessed 08.10.2026).

7. Otsu N. A Threshold Selection Method from Gray-Level Histograms // IEEE Transactions on Systems, Man, and Cybernetics. – 1979. – Vol. 9. – No. 1. – pp. 62–66. – DOI: 10.1109/tsmc.1979.4310076.

8. Ioffe S., Szegedy C. Batch Normalization: Accelerating Deep Network Training by Reducing Internal Covariate Shift // Proceedings of Machine Learning Research. – 2015. – Vol. 37. – pp. 448–456. – URL: https://proceedings.mlr.press/v37/ioffe15.html (accessed 08.10.2026).

9. Simonyan K., Zisserman A. Very Deep Convolutional Networks for Large-Scale Image Recognition // International Conference on Learning Representations. – 2015. – URL: https://arxiv.org/abs/1409.1556 (accessed 08.10.2026).

10. Tan M., Le Q. EfficientNet: Rethinking Model Scaling for Convolutional Neural Networks // Proceedings of Machine Learning Research. – 2019. – Vol. 97. – pp. 6105–6114. – URL: https://proceedings.mlr.press/v97/tan19a.html (accessed 08.10.2026).